Devon Van Tussenbrook

CS 3705 – Protocol Analysis

“Troubleshooting Airplay Connectivity with Roku via Wireshark Network Analysis”

Prof.Ashley Light

7/25/2025

As an introduction to this project you must understand my problem. Late at night as I wind down before bed I like to enjoy people playing videogames live as it has always been relaxing for me. I open up my favorite app twitch.tv to watch but my arm gets tired from holding my phone as I wind down. My tv uses an old roku and it doesn’t have twitch.tv as an app so I must use airplay from my phone. This works most of the time however there is a exactly ten minute window with which my phone can not connect to airplay to my roku. I know this is the case because if you don’t touch your roku for 10 minutes it defaults to a screensaver. I use this to know when my connection will work. Then it connects and works as it should however this problem still exists and intrigues me. I found one solution is to reset my connection to wifi if I’m desperate to fix the problem immediately. This led me to believe that it is some sort of connection issue that I may be able to read from wireshark as I learned about it. As I was also learning about the final project it made for a perfect analysis that I could right about.

Therefore this project is exactly that, this is my process of learning about this issue from a perspective of network-related delays, device limitations, and protocol misconfigurations. This project is relevant because many people rely on communication between devices all over their homes and living spaces. When things go wrong it is easy to get frustrated and quickly leave or even disregard devices or applications. In this project I analyze such an issue in an attempt to further learn about it and perhaps even come to a solution. This will lead to an experience that may help me many times in my future as any day you can device communication issues. It could also help me in future career endeavors as IT workers deal with similar situations everyday.

Before we begin however let us acknowledge expected protocols that we anticipate seeing and analyzing in regards to this project. This first is mDNS which stands for multicast Domain name Service. This is going to be used by Airplay and Roku to advertise them selves on the local network in order to form a connection. We may also want to keep and eye out for SSDP which stand for Simple Service Discovery Protocol. SSDP is a part of UPnP and can also sometimes be used by Roku during its discovery. We also have TCP/UDP which are going to the majority of transport for the communication between the devices. Finally we want to look for HTTP/HTTPS which are used for streaming services and communication with other servers.

While I was eager to get started on this project I was informed that it may turn out to not be so easy as I might need a device that supports promiscuous or monitor mode in order to see the traffic I needed to see. While I was not able to obtain this recommendation I still believe I found interesting and relevant data from my captures. I started by waiting for the perfect night when all my family was asleep to limit interference and background noise. I perfomed multiple captures too isolate functioning and dysfunctional situations. I first started with a capture of isolation with no activity to see what I saw and test running a capture on my own network. Then I got a capture of the roku boot sequence. I also then proceeded to capture the ten minute window where connection failed with 5 attempts at connecting airplay via my phone to the roku. I then proceeded to do the same connecting with my roku and airplay once the ten minute window had passed. This led to incredible data that I found fascinating to sift through.

Sifting through this data was a lot though as it had much too look through. It was interesting seeing many of the processing going on through my network even at this late night. While looking through only then I realized that I had accidentally left my spotify application on going through some of the captures which led to some connection packets. It was important to keep special attention on the timestamps, source/destination IP’s, and service announcements. In my working vs not working capture files I sorted by ip.src being the Ip of my iphone to narrow down the file to what I wanted too see. In the working file you can clearly see a wake and discovery packet at packet 218 and no further requests showing that it worked. In the not working file you can see this same packet information being sent multiple times mirroring the failures as I try to connect at packets 470, 478, 490, 502, 598, 602 and various other times which added up to two each for each connect attempt to the Roku using airplay. Sadly however you could not see the further expected connections that you might see if my capture device had promiscuous or monitor mode.

What would we expect to see if I had this feature working during my capture. Well in the working file we would see a order of events that would go as follows. First we would see the service discovery, followed by the TCP handshake, then they would start a RTSP Session start, then a TLS handshake, with no rejection signals and a wake query which we saw in our working capture. With the failures we could pinpoint where something could be going wrong better than we can now. We would see the same query and repeated queries because it went wrong. We could see a failure of connection at the TCP handshake. We could see an error or lack of response at the RTSP session. There could be a lack of a server hello at the TLS handshake. We could see many rejection signals as we retry.  Finally we do actually see many wake and discovery packets showing the device trying to connect to the Roku, but it isn’t accepting.

In conclusion, I still have my issue and don’t know the exact root cause but I still learned a lot from this experience. My best guess is it seems to be a delayed service discovery failure of my roku after a cold boot. I will have to further continue my investigation with monitor mode enabled too truly discover the issue at hand. I still learned a lot of my issue and various fields such as network troubleshooting, Wireshark usage, and wireless service discovery. I hope to continue my knowledge of these fields in the future of my various careers as it was a blast to take my first solo attempt at them.